3️⃣ Refined Planning Phase (Clean & MECE)

Planning Phase Overview

The project is intentionally divided into four tightly scoped layers, each independently testable.

Layer 1 — Data & Signal Foundation

Objective: Turn raw PU signals into analysable windows.

- Input: PU bearing vibration signals

- Windowing:

  - Fixed-size windows (e.g. 1s, 50% overlap)

- Output:

  - Time-domain windows

  - Frequency-domain representations

✔ Success criterion:

You can load a PU file and reliably produce FFT & spectrogram plots.

Layer 2 — Detection (Is Something Wrong?)

Objective: Detect deviation from healthy behavior.

Baseline-first approach (recommended):

- Statistical metrics (RMS, kurtosis, crest factor)

- Z-score or threshold-based anomaly score

Optional upgrade:

- Autoencoder trained on healthy data only

✔ Success criterion:

Healthy vs faulty segments show a clear anomaly score separation.

Layer 3 — Diagnosis (What Is Likely Wrong?)

Objective: Identify fault type with probabilities.

- Input: anomalous windows only

- Model options:

  - Spectrogram → small CNN

  - Feature vector → classical ML (XGBoost / SVM)

- Output:

  - Fault class probabilities (e.g. inner race, outer race, ball fault)

✔ Success criterion:

Model outputs top fault + confidence, not just a label.

Layer 4 — Agent & Explanation (Why + What Next?)

Objective: Turn raw ML outputs into an engineering decision.

The agent:

- Receives:

  - anomaly score

  - class probabilities

- Decides:

  - which evidence tools to run (FFT, envelope, feature trend)

- Produces:

  - structured incident report

Example output:

Status: ALARM
Likely fault: Outer race (82%)
Evidence:
- Envelope spectrum peak increased in fault band
- Kurtosis increased from 2.1 → 4.8
Recommendation:
- Schedule inspection within 24h
- Check lubrication and bearing alignment

✔ Success criterion:

A human maintenance engineer can understand and trust the output.

5️⃣ Why This Is a Strong “AI Agent” Project (Meta)

This project teaches you:

- Agent thinking (decision orchestration, not just prediction)

- Explainable ML (critical in industry)

- Time-series reasoning

- Product thinking (end-to-end loop)

- Upgrade path:

  - rule-based agent → LLM-based tool-calling agent

  - offline → online

  - local → edge device (Jetson later)

This is exactly aligned with your goal of learning AI agents through real products.

6️⃣ Final Verdict

✅ Your original goal is fundamentally correct

✅ The scope is perfectly chosen

✅ With these adjustments, it becomes:

- clearer

- more defensible

- more “industry-grade”